Multifunctional expansion box

Functions＆features

- Support multiple serial port expansion, can dock more sensors, peripherals, expansion function is stronger.

- Support 4ch RS232 interface, each interface can be set up and work independently.

- Support 1ch RS485 interface.

- Support 4ch switch alarm input, 2ch analog alarm input, 1ch alarm output;

- Support 1ch PWM interface, input and output can be configured.

Technical Parameters

| Project |  | Performance index |
| --- | --- | --- |
| Input interface | power | 5V-12V |
|  | GND | 1piece |
|  | TXD(RS232) | 1 piece |
|  | RXD(RS232) | 1 piece |
| Indicator light | power led | The indicator light comes on when the power is on，When there is data interaction, the lights flicker |
| Output interface | RS232 | 4 piece，independent work |
|  | RS485 | 1 piece |
|  | I/O Alarm input | 4 piece |
|  | Analog quantity alarm input | 2 piece |
|  | Alarm output | 1 piece |
|  | PWM | 1 piece，configurable |
|  | power output | 5V@1A |

Installation matters

- Input interface definition

| PIN | Color | Definition |
| --- | --- | --- |
| 1 | red | 12V/5V |
| 2 | brown | RXD |
| 3 | purple | TXD |
| 4 | black | GND |

- Output interface definition

| PIN | color | definition | PIN | color | definition |
| --- | --- | --- | --- | --- | --- |
| 1 | brown | RX2 | 2 | brown | RX1 |
| 3 | purple | TX2 | 4 | purple | TX1 |
| 5 | brown | RX4 | 6 | brown | RX3 |
| 7 | purple | TX4 | 8 | purple | TX3 |
| 9 | white | ALM_IN2 | 10 | white | ALM_IN1 |
| 11 | white | ALM_IN4 | 12 | white | ALM_IN3 |
| 13 | gray | ALM_OUT | 14 | white | ADC_IN1 |
| 15 | yellow | 485B | 16 | white | ADC_IN2 |
| 17 | orange | 485A | 18 | blue | PWM |
| 19 | black | GND | 20 | pink | 5V |