MX-RLY-12-40 Relay

Product Introduction

Relays are electrical control devices used in vehicles . When the change in input quantity reaches the specified requirement, a relay causes a predetermined step change in the controlled quantity in the electrical output circuit. They are often used in automatic switching systems where small current controls large current operation . Relays play the role of automatic adjustment, safety protection, and circuit conversion in the circuit of the entire automotive equipment .

Features

- 40A contact switching capability

- Contact form 1Z: a set of rotary

- Assembled welding wire for easy use

- Relay pins comply with ISO 7588 PART 1

Core parameters

Electric shock parameters

| Contact Material | silver oxide tin steel |
| --- | --- |
| Contact Forms | 1Z conversion |
| Contact Load | NO: 40A/14VDC |
| Contact Resistance | 50m 2 |
| Max.Switching Current | 150A ON, 40A OFF |
| Electrical Life | 1*10⁵ |
| Mechanical Life | 1*10⁶ |

Coil parameters

| Nominal Volt ( VDC ) | Coil Res. X(1±10%)Ω | Shunt Res. X(1±10%)Ω | Pull-in Volt. ( VDC ) | Release Volt. ( VDC ) | Max Applicable Volt. ( VDC ) |
| --- | --- | --- | --- | --- | --- |
| 12V | 80 |  | 8 | 1.2 | 20.2(23 ℃)15.7(85℃) |

Harness parameters

86 Red Line

85 Black Line

30 87 Yellow Line

Wire gauge: UL1007 #18AWG

Cable length: 20cm

Thread tail: 10MM glue left

Heat shrink sleeve for welding

Product size