Product Model: MX-AHDH-266A-NNP-005H

Features:

◎ 1/2.9″ high-resolution CMOS sensor

◎ 1080P Ultra HD Camera

◎ Supports automatic white balance

◎ Supports low-light night vision down to 0.01 Lux

◎ Horizontal angle of the lens: 165°

◎ Compact design supports better waterproofing 
◎ Multiple mounting holes for quick installation

◎ Appearance color: Black;

Specifications:

| Model | MX-AHDH-266A-NNP-005H |
| --- | --- |
| plan | FH8536+2053 |
| Image sensor | 1/2.9″ CMOS AHD |
| Resolution | 1080P |
| Signal System | PAL system |
| Pixels | 1920 (H) x 1080 (V) |
| lens | 1.58mm lens |
| Horizontal | 165° |
| Minimum Illumination | 0.01 Lux |
| Backlight compensation | automatic |
| Shutter speed | Automatic: 1/50 to 1/100,000 (seconds) |
| White Balance | Automatic white balance tracking |
| Wide dynamic range | none |
| Signal-to-noise ratio | Greater than 55dB (AGC OFF) |
| Video output | 1.0Vp-p, 75Ω |
| power supply | DC12V±10% |
| Power consumption | ≤3W |
| Operating temperature | -20℃ ~ +70℃ |
| Infrared wavelength | / |
| Number of infrared lights | / |
| Infrared light illumination distance | / |
| Infrared light illuminance | / |
| Interface | M12 aircraft carrier bow (with O-ring) |
| External dimensions | 41*40*31 (mm) |

Size specifications :