Model：MX-AHDL-610A-NNP-005H

Product feature

1.Exquisite design, exquisite workmanship, precise lines and rigorous structure, reflecting the highest technology;

2.Integrated structure, shockproof, dust-proof, anti-fog design;

3.Suitable for all kinds of vehicles；

4.Image: positive/mirror image；

5.Color: white;

6.Dedicated aviation interface design；

Specification

| Model | MX-AHDL-610A-NNP-005H |
| --- | --- |
| Image Sensor + DSP | 1/2.7″ CMOS AHD |
| Picture Pixels | 1920(H)*1080(V) |
| Video Frame Rate | PAL: 1080p@25fps, NTSC: 1080p@30fps |
| Video Output | AHD |
| Transmission Distance | More than 500m via 75-3 coaxial cable |
| Minimum Illumination | Day: 0.01Lux, 0Lux (IR ON) |
| Lens | 2.8mm |
| Horizontal angle of view | Optional 90°/100°/105° |
| Sync System | Internal Synchronization |
| Electronic Shutter | PAL: 1/25 -1/50,000 Sec, NTSC: 1/30 -1/60,000 Sec |
| S/N Ratio | More than 50dB |
| Day and Night | Auto (ICR) / Color / B/W |
| Infrared LED | 12mil*Φ5*12pcs LEDs |
| IR Distance | 10-15m |
| OSD Menu | Support |
| UTC | Support |
| OSD/UTC Language | EN, CN, DE, FRA, IT, ES, PL, RU, PT, NL, TR |
| D-WDR | Support |
| Noise Reduction | 3D NR |
| White Balance | Auto / Manual |
| Gain Control | Auto |
| Operation Temperature | -20°C~+70°C / Less than 95%RH (No condensation) |
| Power Supply | DC12V+/-20% |
| Dimension | Φ120*62mm |
| Housing | Ivory color metal case |

Size: