Model: MX-AHDH-701A-NNP-005H

Features:

◎Low power consumption, easy heat dissipation; good color reproduction, clear picture quality;

◎Exquisite, beautiful and fashionable appearance;

◎Fashionable appearance; high strength, highly resistant to impact damage;

◎ Using 9 imported high-power LED lights, shooting distance 10 m;

◎Infrared automatically turns on when the light is lower than 10Lux;

◎Adopt imported lens to prevent water vapor from affecting the image;

◎Appearance color: black ;

◎ Waterproof grade: IP67 ;

◎Specialized aviation interface design .

Specifications:

| Model | MX-AHDH-701A-NNP-005H |
| --- | --- |
| Image Sensor | 1/4″ CMOS AHD |
| Resolution | 1080P/720P |
| Signal system | PAL / NTSC​ |
| Pixel | 1920 ( H) x 1080(V) |
| Lenses | 2.8mm/3.6mm |
| Horizontal angle of view | 100°/80° |
| Minimum illumination | 0.001 ( Lux ) (Infrared on) |
| Backlight compensation | built-in |
| Shutter speed | 1/50~1/10000 seconds ( P ) 1/60-1/10000 seconds ( N ) |
| White Balance | automatic |
| Signal-to-Noise Ratio | Greater than 50db (AGC OFF) |
| Video Output | AHD , amplitude 1Vp-p, 75Ω |
| Power supply | Rated DC12V±10% |
| Power consumption | Infrared light on Infrared light off ≤ 3W |
| Operating temperature | -20 ℃ ~ +70℃ |
| Infrared wavelength | 850nm |
| Number of infrared lights | Φ 5 12LED lamp |
| Infrared light irradiation distance | 5M |
| Infrared light illumination | ＞3Lux |
| Jack | M12 aircraft carrier head (with O -ring) |
| Dimensions | 90mm*77.5mm*62mm |

Dimensions: