Model：MX-AHDH-705A-NNP-005H,2.8mm

Product features:

- Exquisite design, exquisite workmanship, precise circuits and rigorous structure, reflecting the highest technology;

- Wide viewing angle adjustable bracket, which can adjust the angle freely during installation and fixation;

- Using Φ 5 11LED lights with an infrared distance of 10 meters;

- Specialized for monitoring on both sides of professional vehicles;

- Waterproof sealed mechanism, can be used outdoors;

- Integrated structure, shockproof and anti loosening design;

- Suitable for monitoring in various vehicle environments;

- Image: positive/mirror image.

- Appearance color: black;

- Specialized aviation interface design

Specification parameters：

| Image sensor | 1/2.9″ CMOS AHD |  |
| --- | --- | --- |
| Resolution | 720P | 1080P |
| Signal system | PAL/NTSC | PAL/NTSC |
| Pixel | 1280（H) x 720(V) | 1920（H) x 1080(V) |
| Lens | 2.8mm /2.7mm | 2.8mm/2.7mm |
| Horizontal angle of view | 100°/130° | 100°/130° |
| Minimum illumination | 0Lux | 0Lux |
| Backlight compensation | built-in | built-in |
| shutter speed | 1/53~1/100,000（S） | 1/53~1/100,000（S） |
| white balance | Automatically track white balance | Automatically track white balance |
| Signal to noise ratio | >38db (AGC OFF) | >38db (AGC OFF) |
| Video output | 1.0Vp-p,75Ω | 1.0Vp-p,75Ω |
| Power Supply | DC12V±10% | DC12V±10% |
| Power consumption | ≤3W | ≤3W |
| working temperature | -20℃ ~ +70℃ | -20℃ ~ +70℃ |
| Infrared wavelength | 850nm | 850nm |
| IR LED Quantity | Φ5 11LED灯 | Φ5 11LED灯 |
| IR View Distance | 10M | 10M |
| Illuminance when the infrared light is turned on | ＞3Lux | ＞3Lux |
| Jack | VIDEO OUT (Professional aviation interface ) | VIDEO OUT (Professional aviation interface ) |
| Appearance size | 94.68mm*71.58mm*41.5mm | 94.68mm*71.58mm*41.5mm |

SIZE：