Product model：M22-NNNN

Features:

◎ Exquisite design, exquisite workmanship, precise circuit and rigorous structure, reflecting the highest technology;

◎ Equipped with dual camera modeling and detection function ;

◎Meet the needs of most passenger vehicles;

◎Image: positive image/mirror image;

◎Appearance color: black;

◎Specialized aviation interface design;

- System parameters

| project | Equipment parameters | illustrate |
| --- | --- | --- |
|  | Kernel system | L inux |
| system | Operating language | Chinese, English, Spanish (other languages can be customized) |
|  | User interface | C/S operation configuration method |
|  | Operating mode | Remote control, computer client, web |

- Video parameters

| project | Equipment parameters | Indicators |
| --- | --- | --- |
|  | Image sensor | 1/3 CMOS​ |
| Images and videos | Video format | PAL or NTSC standards (optional) |
|  | Horizontal resolution | 700 TV Lines |
|  | Preview function | Single-channel, two-screen splicing; |
|  | Video output | 1-channel video output (CVBS) |
|  | OSD | Images overlaid with information such as time, date, and passenger flow. |
|  | Compression format | H.264​ |
|  | Lens parameters | 960*576 pixels, 2.8 mm (2.8mm/3.6mm/4mm/6mm optional) |
|  | Resolution | D1 |

- Alarm parameters

| project | Equipment parameters | Indicators |
| --- | --- | --- |
| Call the police | Enter | Two alarm I/O inputs, configurable for low-level alarms below 1V or high-level alarms above 4V . |
|  | Functional alarm | Video loss, video obstruction, overload alarm |

- Communication interface

| project | Equipment parameters | Indicators |
| --- | --- | --- |
| Communication interface | RS485 interface | One RS485 interface, supporting protocol integration and custom protocol development. |
|  | RJ45 | One RJ45 interface, which can be used for testing, copying, setting, recording, upgrading, and transmission. |

- Electrical parameters

| project | Equipment parameters | Indicators |  |
| --- | --- | --- | --- |
| Power supply and power consumption | Power input | 8V～36V. Please ensure your vehicle's power supply is within this range before use. Prolonged use above 36V will damage the machine. |  |
|  | Power on/off | Supports AC boot and delayed shutdown. |  |
|  | ACC (Car key signal) | ≤4 V | Power off |
|  |  | ≥5 V | Power on |
|  | Output voltage | +12 V @0.5 A |  |
|  | Power Consumption | Normal operating conditions <4 W |  |

- Other parameters

| project | Equipment parameters | Indicators |  |
| --- | --- | --- | --- |
| Other | Work environment | Temperature | -20 ℃ ~ 70 ℃ |
|  |  | Humidity | 10 % ~ 90 % (non-condensing) |
|  | Heat dissipation | No active cooling, using passive structural cooling. |  |
|  | Size | 133X50X59.65mm (excluding mounting bracket) |  |
|  | Total weight | Approximately 0.45 kg​ |  |
|  | Statistical precision | > 95 % |  |

- Specifications and dimensions