Product introduction

M15Hv is a high-performance, high- expansion product specially developed for in-vehicle video surveillance and remote video surveillance. etc. technology. M15Hv has a compact structure, elegant appearance, and easy installation and use. It is a basic product for lightweight applications. It is suitable for public safety, long-distance passenger transportation, road inspection and other automotive mobile monitoring fields.

Product advantages

- The in-vehicle interface can operate reliably and stably in complex environments

- Video encoding H.265, H.264 optional

- Support 4G , GPS function module

- Support one SD card

- Abundant peripheral interfaces

- Support external UPS battery life

- High scalability

Core parameters

| Project |  | Parameter Description |
| --- | --- | --- |
| Audio and video input | AHD video input | External 4 * 1080P |
|  | Video standards | 30FPS |
|  | Audio Input | 2 |
| Audio and video output | CVBS output | 1 |
|  | VGA output | 1 (optional) |
|  | Audio output | 1 |
| Audio and video parameter encoding | Video encoding | H.264/H.265 |
|  | Video resolution | 1080P/720P/D1 |
|  | Audio coding | G.711A |
| Storage Space | SD card | 1 , maximum 1T |
| Wireless network | 4G wireless | 1 SIM card |
| Positioning method | BeiDou /GPS | Support, mixed positioning possible |
| Interface | USB | 1​ |
|  | RS232 | 1 group |
|  | RS485 | 1 group(optional) |
|  | SENSOR | 4 |
|  | SENSOR OUT | 1 |
|  | NET | 1 (optional) |
| Physical specifications | Size | 102.8mm ( L ) *120mm ( W ) *32.23mm ( H ) (excluding tail wire) |
|  | Weight | NW：0.6kg，GW：0.8kg ( with a fluctuation of 5% ) |
| Work Environment | Operating temperature | -20 ～ +70 ℃ |
|  | Operating humidity | 8% ~ 90% (no condensation) |
| Other | Operation Mode | Remote control, mobile phone APP , WEB |

Product Size

Unit: mm

Panel interface

front panel

rear panel

tail line definition