( RFID version )

Product Introduction

The M19Ev series features an integrated design, incorporating front and rear dual cameras, 4G, Wi-Fi, and GPS. While the rear camera records the vehicle's interior environment, the front camera simultaneously records road conditions, offering significant advantages in convenience and safety. The M19Ev series supports mainstream technologies such as H.265 video encoding and GPS/BD hybrid positioning. Optional features include rapid detection and collision detection, making it suitable for onboard or remote monitoring environments in freight vehicles, passenger buses, construction vehicles, sanitation vehicles.

Product Advantages

- Specialized vehicle interface ensures reliable and stable operation in complex environments.

- Video encoding options: H.265 or H.264

- Built-in 4G, WIFI, and GPS modules

- Supports two Micro-SD (TF) cards .

- Built-in front and rear dual cameras

- It has an adhesive side and supports quick installation.

- RFID support (optional)

Core parameters

| Project |  | Parameter Description |
| --- | --- | --- |
| Audio and video input | Video input | Built-in 2* CMOS 1080P cameras ( front and rear dual view ) |
|  | Forward focal length | 3.0MM |
|  | Forward view | HFOV=111° VFOV=59° DFOV=133° |
|  | Intraocular focal length | 2.1MM |
|  | Inner vision | HFOV=130° VFOV=73° DFOV=160° |
|  | Audio input | Built-in 1 microphone |
| Audio and video output | Audio output | Built-in 1-channel speaker |
| Audio and video parameter encoding | Video encoding | H.265/ H.264 |
|  | Video resolution | 1080P/720P/D1 |
|  | Audio encoding | G.711A |
| Storage space | TF card | 2 cards, maximum 512GB *2 |
| Wireless network | 4G wireless | 1 SIM card |
|  | WIFI | 2.4G |
| Positioning method | BeiDou/GPS | Supports mixed positioning |
| interface | RS232 | Group 1 |
|  | Sensor | 1 High-level alarm: 3.3~36V / Low-level alarm: 0~3.3V |
|  | Sensor out | 1 |
|  | ADC | 1 (Analog input, optional) |
| Physical Specifications | Size | 123.6mm (L)*80.5mm (W)*47.7mm (H) |
|  | Weight | NW: 225g GW : 400g (M19Ev standard version) |
| Work environment | Operating temperature | -20 to +70℃ |
|  | Operating humidity | 8%~90% (non-condensing) |
| other | RFID | Built-in card swiping function (ISO15693 protocol , optional ) |
|  | Collision sensing | Post-parking collision detection |
|  | Gravity sensor | Built-in GSensor |
|  | Linked alarm | Alarms triggered by acceleration, rollover, and collision. |

Product Dimensions