Product introduction

M32Hv series is a dedicated vehicle-mounted audio and video surveillance product that uses a high-speed processor. It adopts a waterproof and dustproof structural design and an embedded operating system, integrating IT cutting-edge technologies such as H.264/H.265 video compression / decompression, 4G network, GPS/BD positioning, etc. It has the characteristics of strong earthquake resistance, simple appearance, flexible and convenient installation, comprehensive functions, and high reliability.

Product Advantage

- The interface can operate reliably and stably in complex environments

- Video encoding H.265, H.264 is optional

- Support 4G 、WIFI and GPS functional modules

- Supports a 2.5-inch optional SSD/HDD hard disk

- Support two SD cards

- Rich peripheral interfaces

- Support external UPS battery life

- Waterproof dust-proof

Core parameters

| Project |  | Parameter Description |
| --- | --- | --- |
| Audio and video input | AHD Video Input | External 4 * 1080P |
|  | Video Standards | 30fps |
|  | Audio Input | 4+1 |
| Audio and video output | CVBS output | 1 |
|  | VGA output | 1 (optional) |
|  | Audio Output | 1 |
| Audio and video parameter encoding | Video Encoding | H.264/H.265 |
|  | Video resolution | 1080P/720P/D1 |
|  | Audio Coding | G.711A |
| Storage Space | Solid State Drive | 1, maximum 8TB |
|  | SD Card | 2 sheets , maximum 1T*2 |
| Wireless network | 4G Wireless | 1 SIM card |
|  | WIFI | 2.4/5.8G ( optional) |
| Positioning method | BeiDou /GPS | Support, mixed positioning possible |
| Interface | RS232 | 1 group |
|  | RS485 | 1 group (enhanced version) |
|  | SENSOR | 2/4 (Enhanced Edition) |
|  | SENSOR OUT | 2 (Enhanced version) |
|  | ADC | 2 (Enhanced version) |
|  | USB | 1 ( Enhanced version) |
|  | NET | 1 (Enhanced version) |
|  | MIC | 1 |
| Physical specifications | Size | 154mm ( L ) *148mm ( W ) *39mm ( H ) (excluding tail wire) |
|  | Weight | NW：1.10 kg，GW：1.40 kg (fluctuating 5% up and down) |
| Work Environment | Operating temperature | -20°C to +70°C |
|  | Protection level | 10% ~ 95% (no condensation) |
|  | Protection Rating | IP66 |
| Other | Operation Mode | Remote control, APP , WEB |
|  | G SENSOR | support |

size

Panel& interface

Front panel

WIFI

Rear panel