Product introduction

M51/52Hv is designed for vehicle video monitoring, remote video monitoring development of a high-performance, highly expandable product, featuring a self-ejecting hard drive structure and high-speed processor NT98323 and embedded operating system, integration of IT cutting-edge field H.264/H.265 video compression or decompression, 4G network, GPS/BD positioning technology. It has the characteristics of strong earthquake resistance, simple appearance, flexible and convenient installation, comprehensive function and high reliability.

Product Advantage

- The interface can operate reliably and stably in complex environments

- Video encoding H.265, H.264 is optional

- Support 4G WIFI and GPS functional modules

- Supports a 2.5-inch optional SSD/HDD hard disk

- Supports one SD card

- Rich peripheral interfaces

- Support external UPS battery life

- High scalability

Core parameters

| Project |  | Parameter Description |
| --- | --- | --- |
| Audio and video input | AHD Video Input | M51Hv external 4 * 1080P M52Hv external 8*1080P |
|  | Video Standards | 30FPS (4 channels) / 15FPS (8 channels) |
|  | Audio Input | 5 |
| Audio and video output | CVBS output | 1 |
|  | VGA output | 1(optional) |
|  | Audio Output | 1 |
| Audio and video parameter encoding | Video Encoding | H.264/H.265 |
|  | Video resolution | 1080P/720P/D1 |
|  | Audio Coding | G.711A |
| Storage Space | Harddisk | SSD/HDD, maximum 8T |
|  | SD Card | 1 card, maximum 1T |
| Wireless network | 4G Wireless | 1 SIM card |
|  | WIFI | 2.4/5.8G ( optional) |
| Positioning method | BeiDou /GPS | Support, mixed positioning possible |
| Interface | RS232 | 1 group |
|  | RS485 | 1 group |
|  | SENSOR | 4 |
|  | SENSOR OUT | 2 |
|  | ADC | 2 |
|  | USB | 1 |
|  | NET | 1 |
|  | MIC | 1 |
| Physical specifications | Size | 176.9mm ( L ) *186.8mm ( W ) *54mm ( H ) (without cable shroud) |
|  | Weight | NW：1.4kg，GW:2kg​ (fluctuating 5% up and down) |
| Work Environment | Operating temperature | -20 ～ +70 ℃ |
|  | Operating humidity | 8% ~ 90% (no condensation) |
| Other | Operation Mode | Remote control, APP , WEB |
|  | G SENSOR | Support |

Size

Panel& interface

Front panel

Rear panel