Product Introduction

- The M71Hv portable video transmission system is a highly portable and extensible product developed for drone monitoring, capturing, command, and HDMI-based ultra-high-definition video transmission. The product features an integrated industrial design, incorporating hardware audio-video encoding, 5G/4G networks, GPS & BeiDou positioning, data storage, and lithium battery modules. It can connect to drones, robots, and DV cameras via an HDMI interface, enabling the management end to remotely access ultra-clear real-time video from the operation site.

Product Advantages

- IP65 protection level, suitable for various complex environments

- Video coding H.265, H.264 optional

- Support 5G, 4G, WIFI, GPS function modules

- Support a MicroSD (TF) card storage

- Built-in lithium battery, continuous work for 8 hours

- Support two-way intercom and cluster intercom;

- Comes with an anti-glare status display

- Comes with USB, HDMI and other interfaces

- High scalability

Core Parameters

| project |  | Parameter Description |
| --- | --- | --- |
| Audio and video input | AHD Input | 1 |
|  | HDMI input | 1 |
|  | Audio Input | 1 (Bluetooth) |
|  | Wired MIC | support |
| Audio Output | Voice Output | 1 (Bluetooth) |
|  | Wired headphones | support |
| Audio and video parameter encoding | Video Encoding | H.264/H.265 |
|  | Video resolution | 1080P/720P |
|  | Audio Coding | G711A , G711U , AAC |
| Storage Space | TF card | 1 sheet, maximum 1T |
| Wireless network | 5G/4G Wireless | 2 SIM cards​ ( With built-in 5G or 4G card access port , supports 1.4G private network) |
|  | WIFI | WiFi Client connection / WiFi AP hotspot |
| Positioning method | BeiDou /GPS | Support, mixed positioning possible |
| power supply | Battery capacity | 7.4v 6000mAh lithium battery pack |
|  | Power Management | Key switch , automatic screen saver |
|  | Type-C charging | Support (current >2A ) |
|  | Magnetic charging | support |
| Screen | OLED screen | 1.54 inches |
|  | show | Positioning, network, power, time |
| button | SOS button | 1 |
| interface | External AHD | 1 ( 4- core M6 aviation connector ) |
|  | TYPE-C | Charging / network port |
|  | Mini HDMI | 1 |
| Physical specifications | size | 149mm ( L ) *98.1mm ( W ) *42.6mm ( H ) (without antenna, back clip) |
|  | weight | NW 0.74 kg ( up or down 5%) |
| Work Environment | Operating temperature | -20 ～ +6 0 ℃ |
|  | Operating humidity | 8% ~ 90% (no condensation) |
| other | Remote Control | PC , APP​ |
|  | Support Agreement | GB28181 , private protocol |

Appearance

Buttons and Interfaces

Top Structure

Side Structure

Bottom Structure